Exercise 1: Multi-Site WAN Extension with Redundant Paths

1. Topology Extension

The baseline code must be modified to create a triangular topology with three nodes (n0, n1, n2) and three point-to-point links.

| Site | Node ID | Function | IP Network |
| --- | --- | --- | --- |
| HQ | n0 | Router | 10.1.1.0/24 (to Branch), 10.1.2.0/24 (to DC) |
| Branch | n1 | Router | 10.1.1.0/24 (to HQ), 10.1.3.0/24 (to DC) |
| DC | n2 | Router | 10.1.2.0/24 (to HQ), 10.1.3.0/24 (to Branch) |

Exporter vers Sheets

C++ Code Snippets (Link Creation and IP Assignment)

A. Link Configuration:

C++

// Create NetDeviceContainers for the three links

NetDeviceContainer devices01, devices02, devices12;

// Point-to-Point Helper setup (assuming it's already configured for 5Mbps, 2ms)// P2PHelper p2p;

// Link 1: HQ (n0) <-> Branch (n1)

devices01 = p2p.Install (n0, n1);

// Link 2: HQ (n0) <-> DC (n2)

devices02 = p2p.Install (n0, n2);

// Link 3: Branch (n1) <-> DC (n2)

devices12 = p2p.Install (n1, n2);

B. IP Address Assignment:

C++

Ipv4AddressHelper ipv4;

// Assign IPs for Link 1 (HQ-Branch: 10.1.1.0/24)

ipv4.SetBase ("10.1.1.0", "255.255.255.0");

ipv4.Assign (devices01); // n0 gets 10.1.1.1, n1 gets 10.1.1.2

// Assign IPs for Link 2 (HQ-DC: 10.1.2.0/24)

ipv4.SetBase ("10.1.2.0", "255.255.255.0");

ipv4.Assign (devices02); // n0 gets 10.1.2.1, n2 gets 10.1.2.2

// Assign IPs for Link 3 (Branch-DC: 10.1.3.0/24)

ipv4.SetBase ("10.1.3.0", "255.255.255.0");

ipv4.Assign (devices12); // n1 gets 10.1.3.1, n2 gets 10.1.3.2

2. Static Routing Table Analysis

The goal is to provide redundancy between HQ (n0) and DC (n2). We assume:

HQ → DC Primary Path: Direct Link (10.1.2.0/24)

HQ → DC Backup Path: Via Branch (n1) (10.1.1.0/24 → 10.1.3.0/24)

Symmetric Routing: DC → HQ must follow the same primary/backup structure.

This is achieved by using the Route Metric in the static routing entries (lower metric = preferred path).

| Node | Destination Network | Next-Hop IP | Interface Index | Metric (Weight) | Notes |
| --- | --- | --- | --- | --- | --- |
| HQ (n0) | DC (10.1.3.0/24) | 10.1.2.2 | 2 (DC Link) | 10 | Primary (Direct) |
| HQ (n0) | DC (10.1.3.0/24) | 10.1.1.2 | 1 (Branch Link) | 20 | Backup (via Branch) |
| Branch (n1) | HQ (10.1.2.0/24) | 10.1.1.1 | 1 (HQ Link) | 10 | Primary (Direct) |
| Branch (n1) | DC (10.1.2.0/24) | 10.1.3.2 | 2 (DC Link) | 10 | Primary (Direct) |
| DC (n2) | HQ (10.1.1.0/24) | 10.1.2.1 | 2 (HQ Link) | 10 | Primary (Direct) |
| DC (n2) | HQ (10.1.1.0/24) | 10.1.3.1 | 1 (Branch Link) | 20 | Backup (via Branch) |

Exporter vers Sheets

Note: Interface indices are for illustration and depend on the NS-3 assignment order.

Implementation using Ipv4StaticRouting::AddNetworkRouteTo

The AddNetworkRouteTo method can take a metric (or "priority") parameter. To set the primary route with a metric of 10 and the backup with a metric of 20, the code on n0 would look like this (simplified):

C++

// Get the static routing helper for n0

Ptr<Ipv4StaticRouting> staticRouting0 = Ipv4RoutingHelper::GetStatic<Ipv4StaticRouting> (n0->GetObject<Ipv4> ());

// Primary route to 10.1.3.0/24 via 10.1.2.2 (DC) with metric 10

staticRouting0->AddNetworkRouteTo (Ipv4Address ("10.1.3.0"), Ipv4Mask ("255.255.255.0"), Ipv4Address ("10.1.2.2"), 10);

// Backup route to 10.1.3.0/24 via 10.1.1.2 (Branch) with metric 20

staticRouting0->AddNetworkRouteTo (Ipv4Address ("10.1.3.0"), Ipv4Mask ("255.255.255.0"), Ipv4Address ("10.1.1.2"), 20);

3. Path Failure Simulation

a) Disabling the Primary Link

In NS-3, a link failure is typically simulated by disabling the NetDevice connected to that link on one or both nodes. The primary HQ-DC link is connected to interfaces on n0 (HQ) and n2 (DC).

C++

// Get the NetDevice on the HQ side (n0) connected to the HQ-DC link (devices02)

Ptr<NetDevice> netDevice0_to_2 = devices02.Get(0);

// Schedule the link down event at t=4 seconds

Simulator::Schedule (Seconds (4.0), &NetDevice::SetDown, netDevice0_to_2);

b) Verifying Traffic Flow

To verify that traffic continues to flow via the backup path (HQ → Branch → DC), you would:

Generate long-running traffic (e.g., a BulkSendApplication or extended UdpEchoClient session) between HQ and DC, starting before t=4s and running past t=5s.

Use NS-3's tracing mechanisms (pcap) to capture packets on all three links.

Analyze the PCAP trace:

Before 4s: Packets between HQ and DC will only be seen on the HQ-DC link.

After 4s: Packets between HQ and DC must now be seen on the HQ-Branch link and the Branch-DC link, proving the backup route was automatically selected due to the lower metric route failing.

c) Measuring Latency

The FlowMonitor tool is ideal for comparing path latency.

Install FlowMonitor on all nodes.

Identify the flow between the HQ client and the DC server.

Collect FlowMonitor statistics at the end of the simulation.

Compare:

Primary Path Latency: Flow statistics collected from t=0 to t=4s.

Backup Path Latency: Flow statistics collected from t=4s onwards.

The backup path's average latency should be slightly higher due to the extra hop (Branch router processing time) and two serial link delays instead of one.

4. Scalability Analysis

Static Routes Needed for a Full Mesh

In a network of N routers, where each router needs a static route entry for every other router, the number of entries is:

Total Static Routes=N×(N−1)

For N=10 sites, the number of static routes required would be:

10×(10−1)=10×9=90 static route entries

More Scalable Approach

A much more scalable approach is to use a Dynamic Routing Protocol, specifically OSPF (Open Shortest Path First). OSPF dynamically discovers the topology, calculates the best path, and automatically converges (reroutes) when a link fails, eliminating the need for manual configuration.

OSPF Implementation in NS-3

NS-3 Helper Class: The primary helper class to implement OSPFv2 is Ospfv2Helper.

Key Configuration Steps:

Install OSPF: Instantiate the helper and install the OSPF protocol on all relevant nodes (n0,n1,n2) using: ospf.Install (nodes);.

Define Area: OSPF requires at least one Area. Define an Area ID and assign interfaces to that Area (e.g., the backbone Area 0.0.0.0): ospf.SetAreaId (Ipv4Address ("0.0.0.0"));.

Enable Routers: Use the helper to enable the protocol on the nodes and interfaces: ospf.EnableRouter (interfaces);.

No Manual Routes: Static routes are no longer needed; OSPF will automatically learn and advertise the connected networks and calculate the best path (Shortest Path First - SPF) to all others.

5. Business Continuity Justification

The triangular topology with proper static routing (using metrics) provides a strong Return on Investment (ROI) for the redundant link cost by ensuring business continuity.

Improved Reliability (High Availability): The metric-based configuration guarantees an automatic failover mechanism. If the primary (direct) HQ-DC link fails, traffic is immediately and transparently rerouted via the Branch router. This prevents a complete WAN outage for critical DC services, maintaining operations and revenue.

Load Balancing Potential (Alternative Use): While the current static setup uses the backup link only for failover, the design could easily be modified (e.g., using Equal-Cost Multi-Path - ECMP via dynamic routing) to leverage both links simultaneously for different traffic classes or destination subnets, thereby increasing total available bandwidth and link utilization.

Simplified Troubleshooting through Deterministic Paths: Unlike dynamic protocols that can change paths unexpectedly, the static metric setup enforces a clear, deterministic primary and secondary path. Troubleshooting is simplified because the network engineer knows exactly which path the traffic should be taking under normal and failed conditions, quickly isolating faults.

Exercise 2: Quality of Service Implementation for Mixed Traffic

1. Traffic Differentiation

The baseline UdpEchoClient must be replaced with two custom application flows that model the traffic characteristics and use Differentiated Services Code Point (DSCP) marking for identification.

Traffic Classes and Parameters

| Class | Traffic Type | Packet Size | Interval | Bandwidth | Latency Req. | DSCP Mark |
| --- | --- | --- | --- | --- | --- | --- |
| Class 1 | VoIP-like (RTP) | 160 bytes | 20 ms | 64 kbps (approx) | Strict | EF (46) |
| Class 2 | FTP-like (Bulk) | 1500 bytes | Burst/On-Demand | High | Best-Effort | BE (0) |

Exporter vers Sheets

NS-3 Implementation & Tagging

Class 1 (VoIP): Use the OnOffApplication with a small packet size and strict rate/interval on a UDP socket.

Packet Size: SetAttribute ("PacketSize", UintegerValue (160))

Rate: SetAttribute ("DataRate", DataRateValue ("40kbps")) (or similar)

Class 2 (FTP): Use the BulkSendApplication (TCP) or another OnOffApplication with large packets and a high rate to create congestion.

Packet Size: SetAttribute ("PacketSize", UintegerValue (1500))

Rate: SetAttribute ("DataRate", DataRateValue ("10Mbps")) (to exceed the 5Mbps link)

DSCP Tagging (using Ipv4L3Protocol::SendIcmpDestUnreach concept): The packets are marked at the sender (HQ or client). This requires accessing the underlying IP Header of the packet before transmission and setting the Differentiated Services Field (DS field).

C++

// Example of setting the DSCP for Class 1 (VoIP - EF/46)

Ptr<Socket> voipSocket = Socket::CreateSocket (n0, UdpSocketFactory::GetTypeId ());

// Use the NS-3 DscpTag or simply manipulate the Differentiated Services field (Type of Service)// A common approach is using a custom helper to set the Dscp value on the outgoing packet.// For example, using the Ipv4Header's DS field:// packet->GetHeader<Ipv4Header> ()->SetTos (46 << 2); // DSCP is 6 bits

2. Queue Management Implementation

A priority queuing system is required on the congested router interface (e.g., n1 for traffic flowing through it). This is implemented by using a specific Queue Discipline attached to the router's NetDevice.

NS-3 Queueing Discipline: Use the PfifoFastQueueDisc (Priority First-in, First-out Queue Disc). This discipline provides three fixed-priority bands. Traffic mapped to Band 0 is served before Band 1, and so on.

Configuration Parameters & Mapping

Configure the Queue Disc:

C++

// Create the PfifoFastQueueDisc Helper

TrafficControlHelper tch;

tch.SetRootQueueDisc ("ns3::PfifoFastQueueDisc");

Create a Classifier: A PfifoFastQueueDisc classifier must be configured to map incoming traffic based on the packet's DSCP value to the appropriate priority Band.

Class 1 (VoIP, DSCP 46/EF): Map to Band 0 (Highest Priority).

Class 2 (FTP, DSCP 0/BE): Map to Band 2 (Lowest Priority).

Installation on the Router Interface:

C++

// Install the queue disc on the interface of the router (n1) facing the congested link// devices01.Get(1) is the NetDevice on n1

tch.Install (devices01.Get(1));

Note: The actual mapping involves setting the ns3::PfifoFastQueueDisc::DscpMap attribute to define which DSCP values fall into which priority band.

3. Performance Measurement

NS-3 Tools and Metrics

| Tool | Metric Collected | Justification |
| --- | --- | --- |
| FlowMonitor | End-to-End Delay (Latency) | Primary measure for latency-sensitive Class 1 traffic. |
| FlowMonitor | Jitter (Variation in Delay) | Critical for VoIP-like Class 1 traffic quality. |
| FlowMonitor | Packet Loss Ratio | Measures the effectiveness of the priority system under congestion (Class 2 should lose more). |
| Custom Trace Sinks | Per-Queue Delay/Dequeue Count | Attach to the QueueDisc to verify that Class 1 packets spend minimal time in the queue and Class 2 packets are dropped when the queue overflows. |

Exporter vers Sheets

Presenting Comparative Results

The results must clearly demonstrate the prioritization. A comparison table is the best way to present this.

| Traffic Class | Metric | Without QoS (Congested) | With QoS (Congested) |
| --- | --- | --- | --- |
| Class 1 (VoIP) | Avg Latency (ms) | 150 | 20 (Low and stable) |
|  | Avg Jitter (ms) | 120 | 5 (Very low) |
|  | Packet Loss (%) | 10% | <1% |
| Class 2 (FTP) | Avg Latency (ms) | 150 | 500+ (High, due to queuing) |
|  | Packet Loss (%) | 10% | 30%+ (Drops prioritized) |

Exporter vers Sheets

4. Congestion Scenario Testing

Creating Link Congestion

The link is 5 Mbps. Congestion is created by sending a combined rate that significantly exceeds this capacity.

Total Offered Rate: Class 1 (VoIP) ≈ 0.1 Mbps + Class 2 (FTP) ≈ 10 Mbps (High Rate) = ≈ 10.1 Mbps

Congestion: ≈10.1 Mbps offered vs. 5 Mbps capacity (a factor of 2× oversubscription).

Expected Behavior

Without QoS: All traffic experiences high, equal latency, and equal probability of packet loss (FIFO queuing). Class 1 (VoIP) quality would be unacceptable.

With QoS: Class 1 (VoIP) latency and loss will remain low (close to baseline) because it is served first by the priority queue. Class 2 (FTP) latency will increase dramatically, and its packet loss will be high, as its packets are either delayed or dropped to protect Class 1.

Specific Simulation Events and Timing

t=0s: Start all traffic generators (VoIP and FTP).

t=1s: The link becomes congested.

t=1s to t=5s: Observe metrics.

Without QoS: All flow metrics are poor.

With QoS: Class 1 metrics are good; Class 2 metrics are poor.

t=5s: Stop the FTP (Class 2) traffic generator.

t=5s to t=10s: Observe metrics. Class 1 metrics should remain good. Class 2 metrics (if any remaining packets) should improve drastically as the congestion clears.

5. Real-World Implementation Gap

NS-3's simplified models often abstract away hardware complexity and deep protocol analysis.

Hardware-Based Shaping/Policing:

Challenge: Real-world QoS is often implemented in specialized router hardware (ASICs) that perform traffic shaping/policing at line rate with nanosecond precision. NS-3 uses software processing, which has inherent, non-trivial overhead that might skew latency measurements, especially in complex models.

Approximation: Implement the shaping/policing logic via NS-3's TokenBucketQueueDisc or custom trace sinks to simply drop or delay packets after they have been processed, effectively simulating the hardware limit without modeling the silicon itself.

Deep Packet Inspection (DPI) for Application Recognition:

Challenge: Real routers use DPI to identify applications (e.g., Skype vs. Zoom video) by analyzing application-layer payload characteristics and behavioral patterns, not just DSCP or port numbers. NS-3's application layer is often simplified, and the simulator generally avoids payload inspection for performance.

Approximation: Use port numbers or socket types as the primary classifier (e.g., all traffic on UDP port 5004 is "Video"). For the simulation, this is sufficient, provided the student manually assigns the correct port numbers to the desired traffic applications.

Context-Aware Congestion Avoidance (e.g., WRED with ECN):

Challenge: Real routers use complex algorithms like Weighted Random Early Detection (WRED) combined with Explicit Congestion Notification (ECN), where the drop probability is based on the average queue size and traffic weight. The implementation details of these in specific hardware are proprietary.

Approximation: Implement a simpler RedQueueDisc (Random Early Detection) which is available in NS-3, and use its basic configuration parameters (min threshold, max threshold, max probability) to approximate the behavior of WRED, focusing on early warning instead of hardware-specific weighting.

Exercise 3: WAN Security Integration and Attack Simulation

1. IPsec VPN Implementation Design

NS-3 historically does not have a full, native IPsec VPN module that implements the entire IKE/AH/ESP protocol stack with cryptographic operations. Therefore, it must be approximated for a conceptual design.

Approximation Method: Network Layer Tunneling

NS-3 Modules/Helpers: The design would rely on the InternetStackHelper and potentially custom classes extending the Ipv4L3Protocol to simulate the encapsulation process.

Security Association (SA) Configuration:

SAs would be represented by a manual configuration table on n0 and n2 (HQ and DC).

The table would map Source/Destination Network → Encryption Key, SPI (Security Parameter Index), and Next-Hop Interface.

When n0 receives a packet for n2's network, it checks the SA table and triggers the encapsulation function.

Implementation Steps

Define a Custom IPsecTunnel Class: This class, attached to the router nodes, handles packets destined for the remote secure network.

Encapsulation/Tunneling: The IPsecTunnel prepends a new IP header (Outer IP) with the tunnel endpoint addresses (n0's interface IP → n2's interface IP). The original packet (Inner IP) is the payload.

Overhead Simulation: The encapsulation/decapsulation process simulates the encryption overhead:

Increased Latency: Add a fixed delay (e.g., 0.1ms) to the packet processing time within the IPsecTunnel::Send and IPsecTunnel::Receive functions.

Reduced Throughput: Increase the packet size by the expected IPsec overhead (AH/ESP headers + padding). For ESP in Tunnel Mode, this is typically ≈50−60 bytes.

Expected Performance Overhead

Increased Latency: Minimal, due to the simulated cryptographic processing time.

Reduced Throughput: Moderate, due to the increased packet size (header overhead), which reduces the effective data rate over the fixed 5 Mbps link.

2. Eavesdropping Attack Simulation

a) Tracing/Packet Capture Mechanisms

The eavesdropper is modeled by a node connected to the shared WAN link (e.g., a tap on the point-to-point link).

NS-3 Method: Use the PcapHelper to enable packet capture on the NetDevice representing the WAN link (e.g., the HQ-DC link device).

Code:

C++

// Assuming devices02 is the NetDeviceContainer for the HQ-DC link

pcap.EnablePcapAll ("wan-tap", devices02);

This generates a .pcap file which can be opened with tools like Wireshark, simulating an attacker sniffing the wire.

b) Extracted Sensitive Information (Unsecured Link)

For the unsecured UdpEchoClient packets, the following could be extracted from the sniffed PCAP trace:

Source and Destination IP Addresses: (e.g., Client IP → Server IP)

Source and Destination Ports: (e.g., UDP port 9)

Application Data: The full payload of the UdpEchoClient (e.g., "Hello world" plus the sequence number/timestamp).

DNS Queries/Other Traffic: Any other application traffic (unencrypted) flowing over the link would be readable.

c) Demonstrating IPsec Effectiveness

Run the Eavesdropping Simulation with the IPsec approximation enabled (from Q1).

Capture a new PCAP trace.

Analysis: The trace will show the Outer IP Headers (with n0→n2 tunnel endpoint IPs), but the Inner IP Header and the UdpEcho payload will be opaque (encrypted payload), proving that the IPsec tunnel successfully protected the confidentiality of the application data.

3. DDoS Attack Simulation

The goal is to overload the DC server (n2) and the intervening WAN link with malicious traffic, measuring the impact on a legitimate flow.

a) Creating Malicious Client Nodes

Add multiple new client nodes (e.g., n3,n4,n5) to the network, connected via another router or directly to the HQ router (n0).

Install Traffic Generator: On each malicious node (n3,n4,n5), install a high-rate OnOffApplication as a client.

b) Traffic Pattern (UDP Flood)

A simple, effective pattern is a UDP Flood targeting the DC server (n2). This is a low-overhead, high-volume attack.

Target: Server's IP address on n2.

Protocol/Socket: UDP

Rate: Set the DataRate attribute on the OnOffApplication to a very high value (e.g., 20 Mbps per attacker).

Timing: Start the attack at t=2 seconds and run it for a set duration.

c) Measuring Impact on Legitimate Traffic

Establish a Legitimate Flow: Use a low-rate UdpEchoClient from n0 to n2 (the flow to be protected), starting at t=0.

Use FlowMonitor:

Before Attack (t=0 to t=2s): Legitimate flow shows low latency and 0% packet loss.

During Attack (t=2s onwards): The legitimate flow's metrics must show a drastic degradation:

Packet Loss: High percentage of packets for the legitimate flow are dropped due to the WAN link buffer filling up with attack traffic.

Latency: Significant increase in end-to-end delay (if any packets get through) due to queuing delay.

4. Defense Mechanisms

a) Rate Limiting (on n0 and n1 Egress/Ingress)

NS-3 Implementation: Use a TokenBucketQueueDisc on the interfaces of the routers (n0 and n1) that face the attacker nodes or the WAN link.

Mechanism: Configure the token bucket with a rate limit (e.g., 5 Mbps for all outbound traffic). Traffic exceeding the token rate is dropped.

Limitation: It is non-discriminatory. It limits all traffic, including legitimate high-rate flows, unless combined with a classifier to only rate-limit the attack traffic.

b) Access Control Lists (ACLs) to Block Malicious Traffic

NS-3 Implementation: ACLs are implemented by adding a packet filter layer to the router's forwarding plane (e.g., the Ipv4L3Protocol). A custom class extending PacketFilter can be used.

Mechanism: The filter checks the Source IP of the attacker nodes (n3,n4,n5) and/or the Destination Port (e.g., UDP port 9) and issues an immediate drop for matching packets before they hit the WAN link.

Limitation: ACLs must be statically configured with the attacker's IP/port. They are ineffective against spoofed source IPs or Zero-Day attacks using new ports.

c) Anycast or Load Balancing to Distribute Attack Traffic

NS-3 Implementation: This is complex and requires multiple DC nodes (n2a,n2b) and the use of ECMP (Equal-Cost Multi-Path) routing or a dedicated load balancer node.

ECMP: Configure two equal-cost static routes to the same network (DC network) from n0, one via n2a and one via n2b. NS-3's forwarding logic will then hash the flows across the two paths.

Mechanism: Attack traffic is distributed across multiple paths/servers, reducing the impact on a single target.

Limitation: Requires significant topology changes (multiple DCs/servers). ECMP in NS-3 typically hashes flows based on the 5-tuple, which might not evenly distribute a flood attack if the flows all share the same source/destination IP.

5. Security vs. Performance Trade-off Analysis

Throughput Reduction with IPsec

Expected Reduction: 5% to 15% depending on the packet size. The reduction is not due to cryptographic processing (which is simulated as a fixed delay) but due to the header overhead (e.g., ≈50 bytes) of the IPsec tunnel mode.

Justification: The 50 bytes of overhead on a 1500-byte packet is 50/1500≈3.3% of the packet. The actual throughput reduction is higher as the network has to spend time transmitting the additional bits, reducing the number of application-layer bytes per second.

Latency Increase with DDoS Protection

Expected Increase: Minimal to Moderate (0-5ms).

Justification: Defense mechanisms like ACLs or simple Rate Limiting are typically implemented very high up in the forwarding plane and add negligible latency (≈0.1ms for a few lookups). Deep Packet Inspection (DPI) or complex Firewall filtering would add more processing delay, potentially up to a few milliseconds. The cost of protection is mainly CPU cycles on the router, not transport delay.

Balanced Security Posture

Based on the simulation insights, a balanced security posture for the company's WAN would be:

Encryption (High Priority): Always enable IPsec VPN across all WAN links. The throughput penalty is minor (5-10%), while the confidentiality gain against eavesdropping is absolute and critical for corporate data.

In-Bound DDoS Protection (Targeted): Implement ACLs/Rate Limiters on the edge router's ingress interface (n0) that filters based on source/destination port and rate. This prevents high-volume attacks from hitting the WAN link, preserving bandwidth for legitimate traffic, with a negligible latency cost.

Dynamic Routing (for Resiliency): Use OSPF/Dynamic Routing (as proposed in Ex 1) over static routing to handle link failures automatically, ensuring the network can recover quickly from physical faults, which is a key component of overall security and resilience.